Libero® SoC v2022.3

SmartFusion® 2 MSS DDR Controller Configuration User Guide

# Introduction

The SmartFusion® 2 MSS has an embedded DDR controller. This DDR controller is intended to control an off-chip DDR memory. The MDDR controller can be accessed from the MSS as well as from the FPGA fabric. In addition, the DDR controller can also be bypassed, providing an additional interface to the FPGA fabric (Soft Controller Mode (SMC)).

To fully configure the MSS DDR controller, you must:

- Select the datapath using the MDDR Configurator.

- Set the register values for the DDR controller registers.

- Select the DDR memory clock frequencies and FPGA fabric to MDDR clock ratio (if needed) using the MSS CCC Configurator.

- Connect the controller’s APB configuration interface as defined by the Peripheral Initialization solution.

You can also build your own initialization circuitry using standalone (not by System Builder) Peripheral Initialization. See SmartFusion2 DDR Controller and Serial High Speed Controller Standalone Initialization Methodology.

Table of Contents

Introduction	1

- MDDR Configurator	3

  - Memory Settings	3

  - Fabric Interface Settings	3

  - I/O Drive Strength (DDR2 and DDR3 only)	4

  - I/O Standard (LPDDR only).	4

  - I/O Calibration (LPDDR only)	4

- MDDR Controller Configuration	5

  - MSS DDR Control Registers	5

  - Importing DDR Configuration Files	9

  - Exporting DDR Configuration Files	10

  - Firmware.	11

  - MSS DDR Configuration Path	12

- Port Description	13

  - DDR PHY Interface.	13

  - Fabric Initiator AXI Bus Interface.	14

  - Fabric Initiator AHB0 Bus Interface.	15

  - Soft Memory Controller Mode AXI Bus Interface	17

  - Soft Memory Controller Mode AHB0 Bus Interface	19

- Revision History	20

Microchip FPGA Support.	21

Microchip Information	21

The Microchip Website.	21

Product Change Notification Service	21

Customer Support.	21

Microchip Devices Code Protection Feature	21

Legal Notice	22

Trademarks.	22

Quality Management System.	23

Worldwide Sales and Service	24

# MDDR Configurator

The MDDR Configurator is used to configure the overall datapath and the external DDR Memory Parameters for the MSS DDR controller.

#### Figure 1-1. MDDR Configurator Overview

The General tab sets your Memory and Fabric Interface settings, as shown in the preceded figure.

## Memory Settings

Enter the DDR Memory Settling Time. The DDR memory requires this time to initialize. The default value is 200 µs. See the DDR Memory datasheet for the correct value to enter.

Use Memory Settings to configure your memory options in the MDDR.

    - Memory Type: LPDDR, DDR2, or DDR3.

    - Data Width: 32-bit, 16-bit, or 8-bit.

    - SECDED Enabled ECC ON or OFF.

    - Arbitration Scheme: Type-0, Type -1, Type-2, or Type-3.

    - Highest Priority ID: Valid values are from 0 through 15.

    - Address Width (bits): See the DDR Memory datasheet for the number of row, bank, and column address bits for the LPDDR, DDR2, or DDR3 memory you use. select the pull-down menu to choose the correct value for rows/banks/columns as per the datasheet of the LPDDR, DDR2, or DDR3 memory.

Important: The number in the pull-down list refers to the number of address bits, not the absolute number of rows, banks, and columns. For example, if your DDR memory has 4 banks, select 2 (22 = 4) for banks. If your DDR memory has 8 banks, select 3 (23 = 8) for banks.

## Fabric Interface Settings

By default, the hard Arm® Cortex®-M3 processor is setup to access the DDR Controller. You can also allow a fabric Initiator to access the DDR Controller by enabling the Fabric Interface Setting check box. In this case, you can choose one of the following options:

    - Use an AXI Interface: The fabric Initiator accesses the DDR Controller through a 64-bit AXI interface.

    - Use a Single AHBLite Interface: The fabric Initiator accesses the DDR Controller through a single 32-bit AHB interface.

    - Use two AHBLite Interfaces: Two fabric Initiator access the DDR Controller using two 32-bit AHB interfaces. The configuration view (Figure 1-1) updates according to your Fabric Interface selection.

## I/O Drive Strength (DDR2 and DDR3 only)

Select one of the following drive strengths for your DDR I/Os:

    - Half Drive Strength

    - Full Drive Strength

Libero® SoC sets the DDR I/O Standard for your MDDR system based on your DDR Memory type and I/O Drive Strength as shown in the following table.

#### Table 1-1. I/O Drive Strength and DDR Memory Type

| DDR Memory Type | Half Strength Drive | Full Strength Drive |
| --- | --- | --- |
| DDR3 | SSTL15I | SSTL15II |
| DDR2 | SSTL18I | SSTL18II |
| LPDDR | LPDRI | LPDRII |

## I/O Standard (LPDDR only)

Select one of the following options:

    - LVCMOS18 (Lowest Power) for LVCMOS 1.8V I/O standard. Used in typical LPDDR1 applications.

    - LPDDRI

Important: Before you choose this standard, make sure that your board supports this standard. You must use this option when targeting the M2S-EVAL-KIT or the SF2-STARTER-KIT boards. LPDDRI I/O standards require that an IMP_CALIB resistor is installed on the board.

## I/O Calibration (LPDDR only)

Choose one of the following options when using LVCMOS18 I/O standard:

    - ON

    - OFF (Typical)

Calibration ON and OFF optionally controls the use of an I/O calibration block that calibrates the I/O drivers to an external resistor. When OFF, the device uses a preset I/O driver adjustment.

When ON, this requires a 150 Ω IMP_CALIB resistor to be installed on the PCB.

This is used to calibrate the I/O to the PCB characteristics. However, when set to ON, a resistor must be installed or the memory controller will not initialize.

For more information, see AC393: SmartFusion2 and IGLOO2 Board Design Guidelines Application Note and

UG0446: SmartFusion2 and IGLOO2 FPGA High Speed DDR Interfaces User Guide.

# MDDR Controller Configuration

When you use the MSS DDR Controller to access an external DDR Memory, the DDR Controller must be configured at runtime. This is done by writing configuration data to dedicated DDR controller configuration registers. This configuration data is dependent on the characteristics of the external DDR memory and your application. This section describes how to enter these configuration parameters in the MSS DDR controller configurator and how the configuration data is managed as part of the overall Peripheral Initialization solution.

## MSS DDR Control Registers

The MSS DDR Controller has a set of registers that need to be configured at runtime. The configuration values for these registers represent different parameters, such as DDR mode, PHY width, burst mode, and ECC. For complete details about the DDR controller configuration registers, see SmartFusion2 DDR Controller and Serial High Speed Controller Initialization Methodology.

### MDDR Registers Configuration

Use the Memory Initialization (Figure 2-1, Figure 2-2, and Figure 2-3), and Memory Timing (Figure 2-4) tabs to enter parameters that correspond to your DDR Memory and application. Values you enter in these tabs are

automatically translated to the appropriate register values. When you click a specific parameter, its corresponding register is described in the Register Description pane (lower portion in Figure 1-1).

### Memory Initialization

The Memory Initialization tab allows you to configure the ways you want your LPDDR, DDR2, or DDR3 memories initialized. The menu and options available in the Memory Initialization tab vary with the type of DDR memory (LPDDR, DDR2, or DDR3) you use.

See the DDR memory datasheet when you configure the options.

When you change or enter a value, the register description pane gives you the register name and register value that is updated. Invalid values are flagged as warnings.

Following figures show the Initialization tab for LPDDR, DDR2, and DDR3, respectively.

#### Figure 2-1. MDDR Configuration—Memory Initialization Parameters (LPDDR)

#### Timing Mode

Select 1T or 2T Timing mode. In 1T (the default mode), the DDR controller can issue a new command on every clock cycle. In 2T timing mode, the DDR controller holds the address and command bus valid for two clock cycles. This reduces the efficiency of the bus to one command per two clocks, but it doubles the amount of setup and hold time.

#### Partial-Array Self Refresh (LPDDR only)

This feature is for power-saving for the LPDDR. Select one of the following for the controller to refresh the amount of memory during a self refresh:

      - Full array: Banks 0, 1, 2, and 3.

      - Half array: Banks 0 and 1.

      - Quarter array: Bank 0.

      - One-eighth array: Bank 0 with row address MSB = 0.

      - One-sixteenth array: Bank 0 with row address MSB and MSB-1 both equal to 0.

For all other options, refer to your DDR Memory datasheet when you configure the options.

#### Figure 2-2. MDDR Configuration—Memory Initialization Parameters (DDR2)

#### Figure 2-3. MDDR Configuration—Memory Initialization Parameters (DDR3)

### Memory Timing

This tab allows you to configure the Memory Timing parameters. See the datasheet of your LPDDR, DDR2, or DDR3 memory when configuring the Memory Timing parameters.

When you change or enter a value, the register description pane gives you the register name and register value that is updated. Invalid values are flagged as warnings.

#### Figure 2-4. MDDR Configuration Memory Timing Tab

Important: The settling time must be added to the desired RESET release to CKE assertion time.

## Importing DDR Configuration Files

In addition to entering DDR Memory parameters using the Memory Initialization and Timing tabs, you can import DDR register values from a file. To do so, click the Import Configuration button and navigate to the text file containing the DDR register names and values. The following figure shows the import configuration syntax.

#### Figure 2-5. DDR Register Configuration File Syntax

Important: If you choose to import register values rather than entering them using GUI, you must specify all necessary register values. See SmartFusion2 and IGLOO2 FPGA High Speed DDR Interfaces User Guide the for details.

## Exporting DDR Configuration Files

You can also export the current register configuration data into a text file. This file will contain register values that you imported (if any) as well as those that were computed from GUI parameters you entered in this dialog.

If you want to undo changes you have made to the DDR register configuration, you can do so with Restore Default. Note: DDR configuration files deletes all register configuration data and you must either re-import or re-enter this data. The data is reset to the hardware reset values.

### 2.3.1	Generated Data

Click OK to generate the configuration. Based on your input in the General, Memory Timing, and Memory Initialization tabs, the MDDR Configurator computes values for all DDR configuration registers and exports these values into your firmware project and simulation files. The exported file syntax is shown in the following figure.

#### Figure 2-6. Exported DDR Register Configuration File Syntax

## Firmware

When you generate the SmartDesign, the following files are generated in the <project dir>/firmware/ drivers_config/sys_config directory. These files are required for the CMSIS firmware core to compile properly and contain information regarding your current design including peripheral configuration data and clock configuration information for the MSS. Do not edit these files manually as they are re-created every time your root design is

re-generated.

    - sys_config.c

    - sys_config.h

    - sys_config_mddr_define.h: MDDR configuration data.

    - sys_config_fddr_define.h: FDDR configuration data.

    - sys_config_mss_clocks.h: MSS clocks configuration

#### Simulation

When you generate the SmartDesign associated with your MSS, the following simulation files are generated in the

<project dir>/simulation directory:

test.bfm Top-level BFM file that is first "executed" during any simulation that exercises the SmartFusion 2 MSS' Cortex-M3 processor. It executes peripheral_init.bfm and user.bfm, in that order.

peripheral_init.bfm Contains the BFM procedure that emulates the CMSIS::SystemInit() function run on the Cortex-M3 before you enter the main() procedure. It essentially copies the configuration data for any peripheral used in the design to the correct peripheral configuration registers and then waits for all the peripherals to be ready before asserting that the user can use these peripherals.

MDDR_init.bfm Contains BFM write commands that simulate writes of the MSS DDR Configuration register data you entered (using the Edit Registers dialog above) into the DDR Controller registers.

user.bfm Intended for user commands. You can simulate the datapath by adding your own BFM commands in this file. Commands in this file will be "executed" after peripheral_init.bfm has completed.

Using the preceding files, the configuration path is simulated automatically. You only need to edit the user.bfm file to simulate the datapath. Do not edit the test.bfm, peripheral_init.bfm, or MDDR_init.bfm files as these files are re-created every time your root design is re-generated.

## MSS DDR Configuration Path

The Peripheral Initialization solution requires that, in addition to specifying MSS DDR Configuration register values, you configure the APB configuration datapath in the MSS (FIC_2). The SystemInit() function writes the data to the MDDR configuration registers via the FIC_2 APB interface.

Tip: If you are using System Builder the configuration path is set and connected automatically.

#### Figure 2-7. FIC_2 Configurator Overview

To configure the FIC_2 interface:

- Open the FIC_2 configurator dialog (see, Figure 2-7) from the MSS configurator.

- Select the Initialize peripherals using Cortex-M3 option.

- Make sure that the MSS DDR is checked, as are the Fabric DDR/SERDES blocks if you are using them.

- Click OK to save your settings. This will expose the FIC_2 configuration ports (Clock, Reset, and APB bus interfaces), as shown in Figure 2-8.

- Generate the MSS. The FIC_2 ports (FIC_2_APB_MASTER, FIC_2_APB_M_PCLK, and FIC_2_APB_M_RESET_N) are now exposed at the MSS interface and can be connected to the CoreConfigP and CoreResetP as per the peripheral initialization solution specification.

For complete details on configuring and connecting the CoreConfigP and CoreResetP cores, see the SmartFusion2 DDR Controller and Serial High Speed Controller Initialization Methodology.

#### Figure 2-8. FIC_2 Ports

# Port Description

The following section lists the ports details.

## DDR PHY Interface

These ports are exposed at the top level of the System Builder generated block. For details, see UG0446: SmartFusion2 and IGLOO2 FPGA High Speed DDR Interfaces User Guide.

#### Table 3-1. DDR PHY Interface

| Port Name | Direction | Description |
| --- | --- | --- |
| MDDR_CAS_N | OUT | DRAM CASN |
| MDDR_CKE | OUT | DRAM CKE |
| MDDR_CLK | OUT | Clock, P side |
| MDDR_CLK_N | OUT | Clock, N side |
| MDDR_CS_N | OUT | DRAM CSN |
| MDDR_ODT | OUT | DRAM ODT |
| MDDR_RAS_N | OUT | DRAM RASN |
| MDDR_RESET_N | OUT | DRAM Reset for DDR3. Ignore this signal for LPDDR and DDR2 Interfaces. Mark it unused for LPDDR and DDR2 Interfaces. |
| MDDR_WE_N | OUT | DRAM WEN |
| MDDR_ADDR[15:0] | OUT | DRAM Address bits |
| MDDR_BA[2:0] | OUT | DRAM Bank Address |
| MDDR_DM_RDQS ([3:0]/[1:0]/[0]) | INOUT | DRAM Data Mask |
| MDDR_DQS ([3:0]/[1:0]/[0]) | INOUT | DRAM Data Strobe Input/Output-P Side |
| MDDR_DQS_N ([3:0]/[1:0]/[0]) | INOUT | DRAM Data Strobe Input/Output-N Side |
| MDDR_DQ ([31:0]/[15:0]/[7:0]) | INOUT | DRAM Data Input/Output |
| MDDR_DQS_TMATCH_0_IN | IN | FIFO in signal |
| MDDR_DQS_TMATCH_0_OUT | OUT | FIFO out signal |
| MDDR_DQS_TMATCH_1_IN | IN | FIFO in signal (32-bit only) |
| MDDR_DQS_TMATCH_1_OUT | OUT | FIFO out signal (32-bit only) |
| MDDR_DM_RDQS_ECC | INOUT | DRAM ECC Data Mask |
| MDDR_DQS_ECC | INOUT | DRAM ECC Data Strobe Input/ Output-P Side |
| MDDR_DQS_ECC_N | INOUT | DRAM ECC Data Strobe Input/ Output-N Side |
| MDDR_DQ_ECC ([3:0]/[1:0]/[0]) | INOUT | DRAM ECC Data Input/Output |
| MDDR_DQS_TMATCH_ECC_IN | IN | ECC FIFO in signal |

| ...........continued |  |  |
| --- | --- | --- |
| Port Name | Direction | Description |
| MDDR_DQS_TMATCH_ECC_OUT | OUT | ECC FIFO out signal (32-bit only) |

Important: Port widths for some ports change depending on the selection of the PHY width. The notation "[a:0]/ [b:0]/[c:0]" is used to denote such ports, where "[a:0]" refers to the port width when a 32-bit PHY width is selected, "[b:0]" corresponds to a 16-bit PHY width, and "[c:0]" corresponds to an 8-bit PHY width.

## Fabric Initiator AXI Bus Interface

The Fabric Initiator AXI Bus Interface ports signals are listed in the following table.

#### Table 3-2. Fabric Initiator AXI Bus Interface

| Port Name | Direction | Description |
| --- | --- | --- |
| DDR_AXI_S_AWREADY | OUT | Write address ready |
| DDR_AXI_S_WREADY | OUT | Write address ready |
| DDR_AXI_S_BID[3:0] | OUT | Response ID |
| DDR_AXI_S_BRESP[1:0] | OUT | Write response |
| DDR_AXI_S_BVALID | OUT | Write response valid |
| DDR_AXI_S_ARREADY | OUT | Read address ready |
| DDR_AXI_S_RID[3:0] | OUT | Read ID tag |
| DDR_AXI_S_RRESP[1:0] | OUT | Read response |
| DDR_AXI_S_RDATA[63:0] | OUT | Read data |
| DDR_AXI_S_RLAST | OUT | Read last This signal indicates the last transfer in a read burst. |
| DDR_AXI_S_RVALID | OUT | Read address valid |
| DDR_AXI_S_AWID[3:0] | IN | Write address ID |
| DDR_AXI_S_AWADDR[31:0] | IN | Write address |
| DDR_AXI_S_AWLEN[3:0] | IN | Burst length |
| DDR_AXI_S_AWSIZE[1:0] | IN | Burst size |
| DDR_AXI_S_AWBURST[1:0] | IN | Burst type |
| DDR_AXI_S_AWLOCK[1:0] | IN | Lock type This signal provides additional information about the atomic characteristics of the transfer. |
| DDR_AXI_S_AWVALID | IN | Write address valid |
| DDR_AXI_S_WID[3:0] | IN | Write data ID tag |
| DDR_AXI_S_WDATA[63:0] | IN | Write data |
| DDR_AXI_S_WSTRB[7:0] | IN | Write strobes |

| ...........continued |  |  |
| --- | --- | --- |
| Port Name | Direction | Description |
| DDR_AXI_S_WLAST | IN | Write last |
| DDR_AXI_S_WVALID | IN | Write valid |
| DDR_AXI_S_BREADY | IN | Write ready |
| DDR_AXI_S_ARID[3:0] | IN | Read address ID |
| DDR_AXI_S_ARADDR[31:0] | IN | Read address |
| DDR_AXI_S_ARLEN[3:0] | IN | Burst length |
| DDR_AXI_S_ARSIZE[1:0] | IN | Burst size |
| DDR_AXI_S_ARBURST[1:0] | IN | Burst type |
| DDR_AXI_S_ARLOCK[1:0] | IN | Lock type |
| DDR_AXI_S_ARVALID | IN | Read address valid |
| DDR_AXI_S_RREADY | IN | Read address ready |
| DDR_AXI_S_CORE_RESET_N | IN | MDDR global reset |
| DDR_AXI_S_RMW | IN | Indicates whether all bytes of a 64-bit lane are valid for all beats of an AXI transfer. 0: Indicates that all bytes in all beats are valid in the burst and the controller must default to write commands 1: Indicates that some bytes are invalid and the controller must default to RMW commands This is classed as an AXI write address channel sideband signal and is valid with the AWVALID signal. Only used when ECC is enabled. |

## Fabric Initiator AHB0 Bus Interface

The Fabric Initiator AHB0 Bus Interface ports signals are listed in the following table.

#### Table 3-3. Fabric Initiator AHB0 Bus Interface

| Port Name | Direction | Description |
| --- | --- | --- |
| DDR_AHB0_SHREADYOUT | OUT | AHBL target ready When high for a write indicates that the MDDR is ready to accept data and when high for a read indicates that data is valid. |

| ...........continued |  |  |
| --- | --- | --- |
| Port Name | Direction | Description |
| DDR_AHB0_SHRESP | OUT | AHBL response status When driven high at the end of a transaction indicates that the transaction has completed with errors. When driven low at the end of a transaction indicates that the transaction has completed successfully. |
| DDR_AHB0_SHRDATA[31:0] | OUT | AHBL read data Read data from the MDDR target to the fabric initiator. |
| DDR_AHB0_SHSEL | IN | AHBL target select When asserted, the MDDR is the currently selected AHBL target on the fabric AHB bus. |
| DDR_AHB0_SHADDR[31:0] | IN | AHBL address Byte address on the AHBL interface. |
| DDR_AHB0_SHBURST[2:0] | IN | AHBL burst length |
| DDR_AHB0_SHSIZE[1:0] | IN | AHBL transfer size Indicates the size of the current transfer (8, 16, or 32 byte transactions only). |
| DDR_AHB0_SHTRANS[1:0] | IN | AHBL transfer type Indicates the transfer type of the current transaction. |
| DDR_AHB0_SHMASTLOCK | IN | AHBL lock When asserted the current transfer is part of a locked transaction. |
| DDR_AHB0_SHWRITE | IN | AHBL write When high indicates that the current transaction is a write. When low indicates that the current transaction is a read. |
| DDR_AHB0_S_HREADY | IN | AHBL ready When high, indicates that the MDDR is ready to accept a new transaction. |
| DDR_AHB0_S_HWDATA[31:0] | IN | AHBL write data Write data from the fabric initiator to the MDDR. |
| DDR_AHB1_SHREADYOUT | OUT | AHBL target ready When high for a write indicates the MDDR is ready to accept data and when high for a read indicates that data is valid. |

| ...........continued |  |  |
| --- | --- | --- |
| Port Name | Direction | Description |
| DDR_AHB1_SHRESP | OUT | AHBL response status When driven high at the end of a transaction indicates that the transaction has completed with errors. When driven low at the end of a transaction indicates that the transaction has completed successfully. |
| DDR_AHB1_SHRDATA[31:0] | OUT | AHBL read data Read data from the MDDR target to the fabric initiator. |
| DDR_AHB1_SHSEL | IN | AHBL target select When asserted, the MDDR is the currently selected AHBL target on the fabric AHB bus. |
| DDR_AHB1_SHADDR[31:0] | IN | AHBL address Byte address on the AHBL interface. |
| DDR_AHB1_SHBURST[2:0] | IN | AHBL burst length |
| DDR_AHB1_SHSIZE[1:0] | IN | AHBL transfer size Indicates the size of the current transfer (8, 16, or 32 byte transactions only). |
| DDR_AHB1_SHTRANS[1:0] | IN | AHBL transfer type Indicates the transfer type of the current transaction. |
| DDR_AHB1_SHMASTLOCK | IN | AHBL lock When asserted the current transfer is part of a locked transaction. |
| DDR_AHB1_SHWRITE | IN | AHBL write When high indicates that the current transaction is a write. When low indicates that the current transaction is a read. |
| DDR_AHB1_SHREADY | IN | AHBL ready When high, indicates that the MDDR is ready to accept a new transaction. |
| DDR_AHB1_SHWDATA[31:0] | IN | AHBL write data Write data from the fabric initiator to the MDDR. |

## Soft Memory Controller Mode AXI Bus Interface

The Soft Memory Controller Mode AXI Bus Interface ports signals are listed in the following table.

#### Table 3-4. Soft Memory Controller Mode AXI Bus Interface

| Port Name | Direction | Description |
| --- | --- | --- |
| SMC_AXI_M_WLAST | OUT | Write last |
| SMC_AXI_M_WVALID | OUT | Write valid |
| SMC_AXI_M_AWLEN[3:0] | OUT | Burst length |
| SMC_AXI_M_AWBURST[1:0] | OUT | Burst type |
| SMC_AXI_M_BREADY | OUT | Response ready |
| SMC_AXI_M_AWVALID | OUT | Write address valid |
| SMC_AXI_M_AWID[3:0] | OUT | Write address ID |
| SMC_AXI_M_WDATA[63:0] | OUT | Write data |
| SMC_AXI_M_ARVALID | OUT | Read address valid |
| SMC_AXI_M_WID[3:0] | OUT | Write data ID tag |
| SMC_AXI_M_WSTRB[7:0] | OUT | Write strobes |
| SMC_AXI_M_ARID[3:0] | OUT | Read address ID |
| SMC_AXI_M_ARADDR[31:0] | OUT | Read address |
| SMC_AXI_M_ARLEN[3:0] | OUT | Burst length |
| SMC_AXI_M_ARSIZE[1:0] | OUT | Burst size |
| SMC_AXI_M_ARBURST[1:0] | OUT | Burst type |
| SMC_AXI_M_AWADDR[31:0] | OUT | Write address |
| SMC_AXI_M_RREADY | OUT | Read address ready |
| SMC_AXI_M_AWSIZE[1:0] | OUT | Burst size |
| SMC_AXI_M_AWLOCK[1:0] | OUT | Lock type This signal provides additional information about the atomic characteristics of the transfer. |
| SMC_AXI_M_ARLOCK[1:0] | OUT | Lock type |
| SMC_AXI_M_BID[3:0] | IN | Response ID |
| SMC_AXI_M_RID[3:0] | IN | Read ID tag |
| SMC_AXI_M_RRESP[1:0] | IN | Read response |
| SMC_AXI_M_BRESP[1:0] | IN | Write response |
| SMC_AXI_M_AWREADY | IN | Write address ready |
| SMC_AXI_M_RDATA[63:0] | IN | Read data |
| SMC_AXI_M_WREADY | IN | Write ready |
| SMC_AXI_M_BVALID | IN | Write response valid |
| SMC_AXI_M_ARREADY | IN | Read address ready |
| SMC_AXI_M_RLAST | IN | Read last This signal indicates the last transfer in a read burst. |

| ...........continued |  |  |
| --- | --- | --- |
| Port Name | Direction | Description |
| SMC_AXI_M_RVALID | IN | Read valid |

## Soft Memory Controller Mode AHB0 Bus Interface

The Soft Memory Controller Mode AHB0 Bus Interface ports signals are listed in the following table.

#### Table 3-5. Soft Memory Controller Mode AHB0 Bus Interface

| Port Name | Direction | Description |
| --- | --- | --- |
| SMC_AHB_M_HBURST[1:0] | OUT | AHBL burst length |
| SMC_AHB_M_HTRANS[1:0] | OUT | AHBL transfer type Indicates that the transfer type of the current transaction. |
| SMC_AHB_M_HMASTLOCK | OUT | AHBL lock When asserted, the current transfer is part of a locked transaction. |
| SMC_AHB_M_HWRITE | OUT | AHBL write When high indicates that the current transaction is a write. When low, indicates that the current transaction is a read. |
| SMC_AHB_M_HSIZE[1:0] | OUT | AHBL transfer size Indicates the size of the current transfer (8, 16, or 32 byte transactions only). |
| SMC_AHB_M_HWDATA[31:0] | OUT | AHBL write data Write data from the MSS initiator to the fabric Soft Memory Controller. |
| SMC_AHB_M_HADDR[31:0] | OUT | AHBL address Byte address on the AHBL interface. |
| SMC_AHB_M_HRESP | IN | AHBL response status When driven high at the end of a transaction indicates that the transaction has completed with errors. When driven low at the end of a transaction indicates that the transaction has completed successfully. |
| SMC_AHB_M_HRDATA[31:0] | IN | AHBL read data Read data from the fabric Soft Memory Controller to the MSS initiator. |
| SMC_AHB_M_HREADY | IN | AHBL ready High indicates that the AHBL bus is ready to accept a new transaction. |

Revision History

# Revision History

The revision history describes the changes that were implemented in the document. The changes are listed by revision, starting with the most current publication.

#### Table 4-1. Revision History

| Revision | Date | Description |
| --- | --- | --- |
| A | 12/2022 | The following is the list of changes in revision A of the document: - The document was migrated to the Microchip template. - The document number was updated to DS50003458 from 50200377. - Updated the following screenshots: - Figure 2-1, Figure 2-2, and Figure 2-3 in 2.1.2. Memory Initialization. - Figure 2-4 in 2.1.3. Memory Timing. - Added: Note in 2.1.3. Memory Timing. |

# Microchip FPGA Support

Microchip FPGA products group backs its products with various support services, including Customer Service, Customer Technical Support Center, a website, and worldwide sales offices. Customers are suggested to visit Microchip online resources prior to contacting support as it is very likely that their queries have been already answered.

Contact Technical Support Center through the website at www.microchip.com/support. Mention the FPGA Device Part number, select appropriate case category, and upload design files while creating a technical support case.

Contact Customer Service for non-technical product support, such as product pricing, product upgrades, update information, order status, and authorization.

- From North America, call 800.262.1060

- From the rest of the world, call 650.318.4460

- Fax, from anywhere in the world, 650.318.8044

# Microchip Information

The Microchip Website

Microchip provides online support via our website at www.microchip.com/. This website is used to make files and information easily available to customers. Some of the content available includes:

- Product Support – Data sheets and errata, application notes and sample programs, design resources, user’s guides and hardware support documents, latest software releases and archived software

- General Technical Support – Frequently Asked Questions (FAQs), technical support requests, online discussion groups, Microchip design partner program member listing

- Business of Microchip – Product selector and ordering guides, latest Microchip press releases, listing of seminars and events, listings of Microchip sales offices, distributors and factory representatives

Product Change Notification Service

Microchip’s product change notification service helps keep customers current on Microchip products. Subscribers will receive email notification whenever there are changes, updates, revisions or errata related to a specified product family or development tool of interest.

To register, go to www.microchip.com/pcn and follow the registration instructions.

Customer Support

Users of Microchip products can receive assistance through several channels:

- Distributor or Representative

- Local Sales Office

- Embedded Solutions Engineer (ESE)

- Technical Support

Customers should contact their distributor, representative or ESE for support. Local sales offices are also available to help customers. A listing of sales offices and locations is included in this document.

Technical support is available through the website at: www.microchip.com/support

Microchip Devices Code Protection Feature

Note the following details of the code protection feature on Microchip products:

- Microchip products meet the specifications contained in their particular Microchip Data Sheet.

- Microchip believes that its family of products is secure when used in the intended manner, within operating specifications, and under normal conditions.

- Microchip values and aggressively protects its intellectual property rights. Attempts to breach the code protection features of Microchip product is strictly prohibited and may violate the Digital Millennium Copyright Act.

- Neither Microchip nor any other semiconductor manufacturer can guarantee the security of its code. Code protection does not mean that we are guaranteeing the product is “unbreakable”. Code protection is constantly evolving. Microchip is committed to continuously improving the code protection features of our products.

Legal Notice

This publication and the information herein may be used only with Microchip products, including to design, test, and integrate Microchip products with your application. Use of this information in any other manner violates these terms. Information regarding device applications is provided only for your convenience and may be superseded by updates. It is your responsibility to ensure that your application meets with your specifications. Contact your

local Microchip sales office for additional support or, obtain additional support at www.microchip.com/en-us/support/ design-help/client-support-services.

THIS INFORMATION IS PROVIDED BY MICROCHIP "AS IS". MICROCHIP MAKES NO REPRESENTATIONS OR WARRANTIES OF ANY KIND WHETHER EXPRESS OR IMPLIED, WRITTEN OR ORAL, STATUTORY OR OTHERWISE, RELATED TO THE INFORMATION INCLUDING BUT NOT LIMITED TO ANY IMPLIED

WARRANTIES OF NON-INFRINGEMENT, MERCHANTABILITY, AND FITNESS FOR A PARTICULAR PURPOSE, OR WARRANTIES RELATED TO ITS CONDITION, QUALITY, OR PERFORMANCE.

IN NO EVENT WILL MICROCHIP BE LIABLE FOR ANY INDIRECT, SPECIAL, PUNITIVE, INCIDENTAL, OR CONSEQUENTIAL LOSS, DAMAGE, COST, OR EXPENSE OF ANY KIND WHATSOEVER RELATED TO THE INFORMATION OR ITS USE, HOWEVER CAUSED, EVEN IF MICROCHIP HAS BEEN ADVISED OF THE POSSIBILITY OR THE DAMAGES ARE FORESEEABLE. TO THE FULLEST EXTENT ALLOWED BY LAW, MICROCHIP'S TOTAL LIABILITY ON ALL CLAIMS IN ANY WAY RELATED TO THE INFORMATION OR ITS USE WILL NOT EXCEED THE AMOUNT OF FEES, IF ANY, THAT YOU HAVE PAID DIRECTLY TO MICROCHIP FOR THE INFORMATION.

Use of Microchip devices in life support and/or safety applications is entirely at the buyer's risk, and the buyer agrees to defend, indemnify and hold harmless Microchip from any and all damages, claims, suits, or expenses resulting from such use. No licenses are conveyed, implicitly or otherwise, under any Microchip intellectual property rights unless otherwise stated.

Trademarks

The Microchip name and logo, the Microchip logo, Adaptec, AVR, AVR logo, AVR Freaks, BesTime, BitCloud, CryptoMemory, CryptoRF, dsPIC, flexPWR, HELDO, IGLOO, JukeBlox, KeeLoq, Kleer, LANCheck, LinkMD, maXStylus, maXTouch, MediaLB, megaAVR, Microsemi, Microsemi logo, MOST, MOST logo, MPLAB, OptoLyzer, PIC, picoPower, PICSTART, PIC32 logo, PolarFire, Prochip Designer, QTouch, SAM-BA, SenGenuity, SpyNIC, SST, SST Logo, SuperFlash, Symmetricom, SyncServer, Tachyon, TimeSource, tinyAVR, UNI/O, Vectron, and XMEGA are registered trademarks of Microchip Technology Incorporated in the U.S.A. and other countries.

AgileSwitch, APT, ClockWorks, The Embedded Control Solutions Company, EtherSynch, Flashtec, Hyper Speed Control, HyperLight Load, Libero, motorBench, mTouch, Powermite 3, Precision Edge, ProASIC, ProASIC Plus, ProASIC Plus logo, Quiet- Wire, SmartFusion, SyncWorld, Temux, TimeCesium, TimeHub, TimePictra, TimeProvider, TrueTime, and ZL are registered trademarks of Microchip Technology Incorporated in the U.S.A.

Adjacent Key Suppression, AKS, Analog-for-the-Digital Age, Any Capacitor, AnyIn, AnyOut, Augmented Switching, BlueSky, BodyCom, Clockstudio, CodeGuard, CryptoAuthentication, CryptoAutomotive, CryptoCompanion, CryptoController, dsPICDEM, dsPICDEM.net, Dynamic Average Matching, DAM, ECAN, Espresso T1S, EtherGREEN, GridTime, IdealBridge, In-Circuit Serial Programming, ICSP, INICnet, Intelligent Paralleling, IntelliMOS, Inter-Chip Connectivity, JitterBlocker, Knob-on-Display, KoD, maxCrypto, maxView, memBrain, Mindi, MiWi, MPASM, MPF, MPLAB Certified logo, MPLIB, MPLINK, MultiTRAK, NetDetach, Omniscient Code Generation, PICDEM, PICDEM.net, PICkit, PICtail, PowerSmart, PureSilicon, QMatrix, REAL ICE, Ripple Blocker, RTAX, RTG4, SAM-

ICE, Serial Quad I/O, simpleMAP, SimpliPHY, SmartBuffer, SmartHLS, SMART-I.S., storClad, SQI, SuperSwitcher, SuperSwitcher II, Switchtec, SynchroPHY, Total Endurance, Trusted Time, TSHARC, USBCheck, VariSense, VectorBlox, VeriPHY, ViewSpan, WiperLock, XpressConnect, and ZENA are trademarks of Microchip Technology Incorporated in the U.S.A. and other countries.

SQTP is a service mark of Microchip Technology Incorporated in the U.S.A.

The Adaptec logo, Frequency on Demand, Silicon Storage Technology, and Symmcom are registered trademarks of Microchip Technology Inc. in other countries.

GestIC is a registered trademark of Microchip Technology Germany II GmbH & Co. KG, a subsidiary of Microchip Technology Inc., in other countries.

All other trademarks mentioned herein are property of their respective companies.

© 2022, Microchip Technology Incorporated and its subsidiaries. All Rights Reserved. ISBN: 978-1-6683-1655-9

Quality Management System

For information regarding Microchip’s Quality Management Systems, please visit www.microchip.com/quality.

Worldwide Sales and Service

| AMERICAS | ASIA/PACIFIC | ASIA/PACIFIC | EUROPE |
| --- | --- | --- | --- |
| Corporate Office | Australia - Sydney | India - Bangalore | Austria - Wels |
| 2355 West Chandler Blvd. | Tel: 61-2-9868-6733 | Tel: 91-80-3090-4444 | Tel: 43-7242-2244-39 |
| Chandler, AZ 85224-6199 | China - Beijing | India - New Delhi | Fax: 43-7242-2244-393 |
| Tel: 480-792-7200 | Tel: 86-10-8569-7000 | Tel: 91-11-4160-8631 | Denmark - Copenhagen |
| Fax: 480-792-7277 | China - Chengdu | India - Pune | Tel: 45-4485-5910 |
| Technical Support: | Tel: 86-28-8665-5511 | Tel: 91-20-4121-0141 | Fax: 45-4485-2829 |
| www.microchip.com/support | China - Chongqing | Japan - Osaka | Finland - Espoo |
| Web Address: | Tel: 86-23-8980-9588 | Tel: 81-6-6152-7160 | Tel: 358-9-4520-820 |
| www.microchip.com | China - Dongguan | Japan - Tokyo | France - Paris |
| Atlanta | Tel: 86-769-8702-9880 | Tel: 81-3-6880- 3770 | Tel: 33-1-69-53-63-20 |
| Duluth, GA | China - Guangzhou | Korea - Daegu | Fax: 33-1-69-30-90-79 |
| Tel: 678-957-9614 | Tel: 86-20-8755-8029 | Tel: 82-53-744-4301 | Germany - Garching |
| Fax: 678-957-1455 | China - Hangzhou | Korea - Seoul | Tel: 49-8931-9700 |
| Austin, TX | Tel: 86-571-8792-8115 | Tel: 82-2-554-7200 | Germany - Haan |
| Tel: 512-257-3370 | China - Hong Kong SAR | Malaysia - Kuala Lumpur | Tel: 49-2129-3766400 |
| Boston | Tel: 852-2943-5100 | Tel: 60-3-7651-7906 | Germany - Heilbronn |
| Westborough, MA | China - Nanjing | Malaysia - Penang | Tel: 49-7131-72400 |
| Tel: 774-760-0087 | Tel: 86-25-8473-2460 | Tel: 60-4-227-8870 | Germany - Karlsruhe |
| Fax: 774-760-0088 | China - Qingdao | Philippines - Manila | Tel: 49-721-625370 |
| Chicago | Tel: 86-532-8502-7355 | Tel: 63-2-634-9065 | Germany - Munich |
| Itasca, IL | China - Shanghai | Singapore | Tel: 49-89-627-144-0 |
| Tel: 630-285-0071 | Tel: 86-21-3326-8000 | Tel: 65-6334-8870 | Fax: 49-89-627-144-44 |
| Fax: 630-285-0075 | China - Shenyang | Taiwan - Hsin Chu | Germany - Rosenheim |
| Dallas | Tel: 86-24-2334-2829 | Tel: 886-3-577-8366 | Tel: 49-8031-354-560 |
| Addison, TX | China - Shenzhen | Taiwan - Kaohsiung | Israel - Ra’anana |
| Tel: 972-818-7423 | Tel: 86-755-8864-2200 | Tel: 886-7-213-7830 | Tel: 972-9-744-7705 |
| Fax: 972-818-2924 | China - Suzhou | Taiwan - Taipei | Italy - Milan |
| Detroit | Tel: 86-186-6233-1526 | Tel: 886-2-2508-8600 | Tel: 39-0331-742611 |
| Novi, MI | China - Wuhan | Thailand - Bangkok | Fax: 39-0331-466781 |
| Tel: 248-848-4000 | Tel: 86-27-5980-5300 | Tel: 66-2-694-1351 | Italy - Padova |
| Houston, TX | China - Xian | Vietnam - Ho Chi Minh | Tel: 39-049-7625286 |
| Tel: 281-894-5983 | Tel: 86-29-8833-7252 | Tel: 84-28-5448-2100 | Netherlands - Drunen |
| Indianapolis | China - Xiamen |  | Tel: 31-416-690399 |
| Noblesville, IN | Tel: 86-592-2388138 |  | Fax: 31-416-690340 |
| Tel: 317-773-8323 | China - Zhuhai |  | Norway - Trondheim |
| Fax: 317-773-5453 | Tel: 86-756-3210040 |  | Tel: 47-72884388 |
| Tel: 317-536-2380 |  |  | Poland - Warsaw |
| Los Angeles |  |  | Tel: 48-22-3325737 |
| Mission Viejo, CA |  |  | Romania - Bucharest |
| Tel: 949-462-9523 |  |  | Tel: 40-21-407-87-50 |
| Fax: 949-462-9608 |  |  | Spain - Madrid |
| Tel: 951-273-7800 |  |  | Tel: 34-91-708-08-90 |
| Raleigh, NC |  |  | Fax: 34-91-708-08-91 |
| Tel: 919-844-7510 |  |  | Sweden - Gothenberg |
| New York, NY |  |  | Tel: 46-31-704-60-40 |
| Tel: 631-435-6000 |  |  | Sweden - Stockholm |
| San Jose, CA |  |  | Tel: 46-8-5090-4654 |
| Tel: 408-735-9110 |  |  | UK - Wokingham |
| Tel: 408-436-4270 |  |  | Tel: 44-118-921-5800 |
| Canada - Toronto |  |  | Fax: 44-118-921-5820 |
| Tel: 905-695-1980 |  |  |  |
| Fax: 905-695-2078 |  |  |  |